Product #1: Vaisala HUMICAP Humidity and Temperature Probe HMP110

Guarantee: 24 months — minimum requirement

- Miniature-size humidity transmitter

- Low power consumption and fast start-up for battery-powered applications

- Measurement range:0 ... 100 %RH; -40 ... +80 °C (-40 ... +176 °F)

- Cable detachable with standard M8 quick connector

- IP65 metal housing

- Optional RS-485 digital output supports Modbus RTU

- ±1.5 %RH measurement accuracy (0 ... 90 %RH)

Measurement Performance

Relative Humidity

Measurement range 			0 … 100 %RH

Accuracy: 1) 2)

at 0 … +40 °C (+32 … +104 °F) 		±1.5 %RH (0 … 90 %RH)

±2.5 %RH (90 … 100 %RH)

at −40 … 0 °C, +40 … +80 °C		±3.0 %RH (0 … 90 %RH)

(−40 … +32 °F, +104 … +176 °F) 		±4.0 %RH (90 … 100 %RH)

Factory calibration uncertainty		±1.1 %RH (0 … 90 %RH)

at +20 °C (+68 °F)			±1.8 %RH (90 … 100 %RH)

Humidity sensor types 			HUMICAP 180R

HUMICAPâ 180V

Stability 				±2 %RH over 2 years

Temperature

Measurement range 			−40 … +80 °C (−40 … +176 °F)

Accuracy (Probes with Analog Output):

at 0 … +40 °C (+32 … +104 °F) 		±0.2 °C (±0.36 °F)

at −40 … 0 °C, +40 … +80 °C		±0.4 °C (±0.72 °F)

(−40 … +32 °F, +104 … +176 °F)

Accuracy (Probes with Digital Output):

at +15 … +25 °C (+59 … +77 °F) 		±0.1 °C (±0.18 °F)

at 0 … +15 °C, +25 … +40 °C		±0.15 °C (±0.27 °F)

(+ 32 … +59 °F, +77 … +104 °F)

at −40 … 0 °C, +40 … +80 °C		±0.4 °C (±0.72 °F)

(−40 … +32 °F, +104 … +176 °F)

Temperature sensor 			Pt1000 RTD Class F0.1 IEC 60751

Calculated Parameters

Measurement range for dew point		−40 … +80 °C (−40 … +176 °F)

temperature

Measurement range for wet bulb		−40 … +80 °C (−40 … +176 °F)

temperature

Measurement range for enthalpy 		−40 … 1540 kJ/kg

(−10 … +660 BTU/lb)

Accuracy at 20 °C (68 °F) and 80 %RH: 1)

Dew Point

at 0 … +40 °C (+32 … +104 °F)

• when dew point depression		±1 °C (±33.8 °F)

< +15 °C (+59 °F)

• when dew point depression		±2 °C (±35.6 °F)

+15 … +25 °C (+59 … +77 °F)

at −40 … 0 °C, +40 … +80 °C		±2 °C (±35.6 °F)

(−40 … +32 °F, +104 … +176 °F)

• when dew point depression

< +15 °C (+59 °F) 3)

Wet Bulb Temperature

at 0 … +40 °C (+32 … +104 °F) 		±0.4 °C (0.72 °F)

(0 … 90 %RH) 				(probes with analog output)

±0.3 °C (0.54 °F)

(probes with digital output)

Enthalpy

at +15 … +25 °C (+59 … +77 °F) 		±1.2 kJ/kg (0.52 BTU/lb)

(0 … 90 %RH) 				(probes with analog output)

±0.9 kJ/kg (0.4 BTU/lb)

(probes with digital output)

Analog Outputs

Accuracy at +20 °C (+68 °F) 		±0.2 % of FS

Temperature dependence 			±0.01 % of FS/°C (±0.006 % of FS/°F)

Mechanical Specifications

IP rating 				IP65

Body thread 				MI2x1 / 10 mm (0.4 in)

Cable connector 				M8 4-pin female (IEC 60947-5-2)

Materials

Body 					Stainless steel (AISI 316)

Grid filter 				Chrome coated ABS plastic

Cable 					Polyurethane or FEP

Weight

Probe 					17 g (0.6 oz)

Probe with 0.3 m (1 ft) cable 		28 g (1 oz)

Operating Environment

Operating temperature 			−40 … +80 °C (−40 … +176 °F)

EMC compliance 			EN 61326-1, industrial environment

Inputs and Outputs

Power consumption 			1 mA average, max. peak 5 mA

Operating Voltage

With 1 V / 2.5 V output 			5 … 28 VDC

With 5 V output 				8 … 28 VDC

With loop power converter 		8 … 28 VDC

With digital output 			5 … 28 VDC

Start-Up Time

HMP110 probes with analog output 	4 s at operating voltage

13.5 … 16.5 VDC

2 s at other valid operating voltages

HMP110 probes with digital output 	1 s

Outputs

2 channels 				0 … 1 VDC / 0 … 2.5 VDC / 0 … 5 VDC /

1 … 5 VDC

1-channel loop-power converter		4 … 20 mA

(separate module, compatible with

humidity accuracy only)

Digital output (HMP110 probes with 	RS-485 2-wire half duplex, supports

digital output) 				Modbus RTU

External Loads

0 … 1 V 				RL min 10 kΩ

0 … 2.5 V /0 … 5 V 			RL min 50 kΩ

Spare Parts and Accessories

4 … 20 mA loop power converter 		UI-CONVERTER-1CB

Mounting bracket for converter 		225979

Plastic M12 installation nuts, pair 		18350SP

USB cable for PC connection 		219690

Probe mounting clamp set, 10 pcs 		226067

Probe mounting flange 			226061

Sensor Protection

Plastic grid 				DRW010522SP

Membrane filter 				DRW010525SP

Stainless steel sintered filter 		HM46670SP

PTFE sintered filter 			DRW244938SP

Stainless steel grid filter ASM212652SP

Connection Cables

Standard 0.3 m (1 ft) 			HMP50Z032SP

Standard 3 m (9.8 ft) 			HMP50Z300SP

+80 °C 1.5 m (+176 °F 5 ft) 		225777SP

+80 °C 3 m (+176 °F 10 ft) 		225229SP

+180 °C 3 m (+356 °F 10 ft) FEP 		226902SP

Connection cable for HM70 		219980SP

Flat extension cable 1 m (3 ft) 1) 		CBL210649S

Product #2: Vaisala WA15 anemometer

Guarantee: 12 months --- minimum requirement

WAC151 Crossarm for Wind Sensors (w. Thermostat)

WAA151 Wind speed sensor WAA151

Transmitter:

Analogoutputsignal (range 0....75m/s):

0...750 Hz square wave

Power supply: 9,5...15VDC

Operating Temp. with shaft heating: -50...55°C

Sensor:

Wind speed:

Measuring range: 0,4...75 m/s

Accuracy: +/-0,17 m/s with characteristic transfer function

WAV151 Wind direction sensor

Product #3: 10.15184.400000 	Weighing precipitation Sensor

Guarantee: 12 months — minimum requirement

| Professional Line | rain[e]400 Weighing precipitation sensor |
| --- | --- |
| Id-No. | 00.15184.004 000 rain[e]400, unheated · 00.15184.404 000 rain[e]400, heated |
| Measuring range | without limitation (0.0025... ∞ mm) |
| Amount of accuracy | ± 0.1 mm or ± 1 % at < 3 mm/min and ± 2 % at ≥ 3 mm/min |
| Amount of Resolution | 0.001 mm (pulse output: 0.01 mm) |
| Intensity range | 0…10 mm/min resp. 0…600 mm/h |
| Intensity of resolution | 0.001 mm/min resp. 0.001 mm/h |
| Output | • SDI-12 • RS-485 (SDI-12 protocol, ASCII protocol, TALKER protocol) • 2 Pulse-Outputs for linearised, bounce-free output signal • Status-Output (configurable, e.g. rain yes/no or heating on/off) • Analogue output 0/4...20 mA (0...2.5/5 V) |
| Collection surface | 400 cm2 |
| Environmental conditions | 0...+70 °C (unheated), -40...70 °C (heated, no icing, no snowdrift) |
| Supply voltage | unheated: 9.8 ... 32 VDC heated: 24VDC / 2 heating circuits 150 W (funnel) and 60 W (outlet / tipping bucket) |
| Power consumption | max. 45 mA at 24 V power supply and analogue output • typ. 7.5 mA at 24 V power supply and deactivated analog output • typ. 12.5 mA at 12 V |
| Target temperature (heating) | +2 °C funnel surface temperature |
| Measuring principle | weighing with automatic self emptying |
| Design | mounting mast Ø 60 mm |
| Dimensions | 311 mm x 256 mm (H x D) |
| Protection class | IP67 |
| Weight | approx. 4 kg |
| Standards | WMO-No. 8 • VDI 3786 Bl. 7 • EN 61000-2, -4 • EN 61000-4-2, -3, -4, -5, -6, -11 • NAMUR NE-21 |
| Accessories (order separately) | 00.15091.600XXX Windshield according to Tretyakov for measuring height 1 m, 1.5 m and 2 m 32.15184.060000 sensor cable, 10 m, 8-polig, M12-Stecker 32.15184.061000 Power supply cable for sensor heating, 1 m |

15184H rain [e] with electronically controlled 2-circuit heating

Measuring principle: mainly with automatic self-emptying

Operating temperature: 0 ... + 70 ° C (unheated)

Auffangfl äche: 200 cm

Amount Range: limitati without one (0.005 ... 8 mm)

Ed amount OLUTION: 0.001 mm (pulse output: 0.01 mm)

Amount accuracy: ± 0.1 mm or ± 1% at <6 mm / min

and ± 2% for> 6 mm / min

Intensity range: 0 ... 20 mm / min and 0 ... 1200 mm / h

Intensity ed OLUTION: 0.001 mm / min and 0.001 mm / h

Intensity accuracy: ± 0.1 mm / min or ± 6 mm / h

Protection class load cell: IP67

Power consumption: max. 45 mA at 24 V supply and analog output •

5 mA at 24 V supply and pulse output

Supply voltage: 9.8 ... 32 V DC

Output signals: · SDI-12 • RS-485 (SDI-12 protocol, ASCII protocol TALKER protocol)

· 2 pulse outputs for linearized, bounce-free output signal

· Status output (confi gurable, eg rain yes / no or heating on / off)

· Analog output 0/4 ... 20 mA (0 ... 2.5 / 5V

• USB cable for configuration; L = 1 m

• user Manual

• Configuration Software rain [e] Commander

32.15184.060000: 10 metres sensor cable — minimum cable lenght

32.15184.061010: 10 metres heating cable — minimum cable lenght

Product #4: CMP11 CMP11 Pyranometer, (Kipp&Zonen)

Guarantee: 12 months — minimum requirement

Spectral range: 285 to 2800 nm

Sensitivity: 7 to 14 µV/W/m²

Response time: ± 5 s

Zero offset A: ± 7 W/m²

Zero offset B: ± 2 W/m²

Directional error (up to 80 ° with 1000 W/m² beam): < 10 W/m²

Temperature dependence of sensitivity (-10 ºC to +40 ºC): ± 1 %

Operating temperature range: -40 °C to +80 °C

Maximum solar irradiance: 4000 W/m²

Field of view: 180 °

Cable 10 metres — minimum cable lenght

Product #5: Digital Barometer

Guarantee: 24 months — minimum requirement

PTB210A  BAROCAP Digital barometer

Features

• 500 … 1100 hPa or 50 … 1100 hPa

pressure ranges with serial output

• Different scalings between

500 … 1100 hPa with

analog output

• Electronics housing IP65

protected against sprayed water

• Accurate and stable measurement

• Traceable calibration

(certificate included)

Measurement Performance

Pressure Range

Serial output 			500 … 1100 hPa

50 … 1100 hPa

Analog output 			500 … 1100 hPa

600 … 1060 hPa

800 … 1060 hPa

900 … 1100 hPa

Serial Output (Units in hPa), Accuracy

Pressure range 			500 … 1100 				50 … 1100

Class A 			Class B

Non-linearity1) 			± 0.10 			± 0.15 		± 0.20

Hysteresis1) 			± 0.05 			± 0.05 		± 0.10

Repeatability1) 			± 0.05			 ± 0.05		 ± 0.10

Calibration			± 0.07 			± 0.15 		± 0.20

uncertainty2)

Accuracy at			± 0.15			 ± 0.20 		± 0.35

+20 °C (+68 °F)3)

Temperature			± 0.20 			± 0.20		 ± 0.40

dependence4)

Total accuracy			± 0.25 			± 0.30		 ± 0.50

-40 … +60 °C

(-40 … +140 °F)3)

Long term stability		± 0.10			 ± 0.10 		± 0.20

(hPa/year)

Analog Output, Accuracy

Non-linearity1)						 ± 0.20 hPa

Hysteresis1) 						± 0.05 hPa

Repeatability1)						 ± 0.05 hPa

Calibration uncertainty2) 					± 0.15 hPa

Accuracy at +20 °C (+68 °F)3) 				± 0.30 hPa

Temperature dependence4) 				± 0.50 hPa

Total accuracy -40 … +60 °C				± 0.60 hPa

(-40 … +140 °F)3)

Long term stability 					± 0.10 hPa/year

1) Defined as the ±2 standard deviation limits of end point non-linearity, hysteresis error, or repeatability

error.

2) Defined as ±2 standard deviation limits of inaccuracy of the working standard including traceability to

international standards.

3) Defined as the root sum of the squares (RSS) of end point non-linearity, hysteresis error, repeatability

error, and calibration uncertainty at room temperature.

4) Defined as ±2 standard deviation limits of temperature dependence over the operating temperature

range.

Operating Environment

Operating temperature 					-40 … +60 °C (-40 … +140 °F)

Operating humidity 					Non-condensing

EMC compliance 					EN61326-1, Generic Environment

Mechanical Specifications

Housing material 					PC Plastic

IP rating, electronics					IP65 (NEMA 4)

IP rating, sensor 						IP53

Instrument weight 					110 g (3.9 oz)

Cable weight 						28 g/m (1.0 oz

Inputs and Outputs

Serial Output

Shutdown 						ON/OFF

Settling time at startup 					2 s

Serial I/O 						RS-232C

RS-232C /TTL (optional)

RS-485, non isolated (optional)

Parity 							None, even, odd

Data bits 						7, 8

Stop bits 						1, 2

Baud rate 						1200, 2400, 4800, 9600, 19200

Response time 						1 s

Resolution 						0.01 hPa (1 measurement/s)

0.03 hPa (10 measurements/s)

Current consumption, normal mode 			< 15 mA (factory setting)

Current consumption, power down				< 0.8 mA

mode

Current consumption, shutdown mode 			0.2 mA

Analog Output

Outputs 							0 … 5 VDC, 0 … 2.5 VDC (order

specified)

Shutdown 						ON/OFF

Response time 						500 ms

Resolution 						300 μV

Measurement rate 					3 measurements/s

Current consumption, normal mode			 < 8 mA

Current consumption, shutdown mode 			0.2 mA

All Models

Max. pressure 						5 000 hPa abs.

Pressure connector 					M5 (10-32) internal thread

Pressure fitting 						Barbed fitting for 1/8 in I.D. tubing

Supply voltage						5 … 28 VDC

(reverse polarity protected),

with RS-232/TTL output

Supply voltage						8 … 18 VD

(reverse polarity protected),

with RS-485 or analog output

Cable: 5 metres — minimum calbe leght

PTB210PZ2	2m cable for PTB210

PTB210RS	RS485 Serial Communication module

215094	DIN rail kit for HMT330/DMT340/MMT330

215109	Rain Shield with Pole Installation Kit

HMT330/DMT340/MMT330/HMT120/130

Product #6: ST01 Hukseflux soil temperature sensor

Guarantee: 12 months — minimum requirement

Measurement range			-60 to +150 °C, cable remaining flexible

Standard sensor				Pt100, IEC 751:1983 class A

Alternative sensors			Pt500, Pt1000

Attainable accuracy	+/- 0.25 °C +/- 0.15 °C with individual calibration (both across -50 to +50 °C)

Moisture protection needle 		IP 68

and cable until wiring exit

Pressure resistance			6 bar

UV resistance				no UV degradation

Standard cable length			3 m

Expected lifetime			>10 yrs (normal field conditions)

Cable material				extruded PTFE

Cable: 5 metres — minimum cable lenght