Specifications — PB13308

The MTi-670 is a GNSS/INS with a small form-factor design for deep integration into your outdoor application. Building on the proven Xsens MTi 600-series technology it enables a robust and easy to use meter level positioning and orientation tracking. It features an interface to an external GNSS receiver so you can efficiently design your application. It is designed for easy integration and seamless interfacing with other equipment. The MTi-670 is supported by the MT Software Suite which includes MT Manager (GUI for Windows/Linux), SDK, example codes and drivers for many platforms including ROS.

Specifications:

- Interfaces: UART, CAN, RS232

- Protocols: Xbus, ASCII (NMEA), CAN

- Output frequency: up to 2kHz

- Built-in self-test: Gyr, Acc, Mag, Baro, GNSS

- Material: PC-ABS

- IP-rating: IP51

- Sizes: 28 x 31.5 x 13 mm

- Weight: 8.9 g

- Input voltage: 4.5 – 24 V

- Consumption < 1 W

- Gyroscope: 2000 deg/s range, 500 Hz bandwidth

- Accelerometer: 10g range, 500 Hz bandwidth

- Magnetometer: +/- 8 G range, 0.25 mG resolution

- Barometer: 300-1250 hPa, +/- 8 Pa relative accuracy