# SIGNAL GENERATOR

- No. of Channels: 2 Channel

- Bandwith 60MHz (or greater)

- Built-in waveforms: Sine, Square, Pulse, Ramp, Noise, Arbitrary waveform

- Sine waves range		1 μHz to 60 MHz (or wider range)

- Square wave range		1 μHz to 25 MHz (or wider range)

- 300 MS/s sampling rate in arbitrary waveforms, 14 bits resolution (or greater)

- Output Amplitude range 1 mVpp – 10 Vpp  (50Ω load) (or wider range)

- Display type 3.95-inch (or greater), color

- Interfaces: USB host, USB device (LAN not necessary)

- modulation modes (AM, FM, PM, ASK, FSK, PSK, PWM)

- 1 M points arbitrary waveforms memory enables (or greater)

- 6 digit resolution frequency counter (or better resolution)

- 3-year warranty (or longer)

# Power Supply

- 2 Channels (or more)

- outputs are galvanically isolated

- Total power output 100 W (or more)

- Maximum power per channel 50 W (or more)

- Current output per channel 5 A (or more)

- Voltage output: 0 V to 30 V (or more)

- Graphic Color display

- Interfaces: USB, LAN

- Constant voltage mode: <0,03%+5 mV

- Constant current mode: <0,03%+200 µA

- Voltage resolution 1 mV (or better)

- Current resolution: 0,1 mA (<1A), 1 mA (>1A) (or better)

- Overvoltage/overcurrent protection for all outputs, FuseLink technology

- Electronic Fuse: linking < 100 us

- EasyArb function for user-definable V/I curves

- EasyRamp for simulating a start-up curve

- sequenced start of channels

- Data logging to USB flash drive

- SCPI compatible

# Hot Plate

- Dimensions incl. pcb holder (L x W x H): 193 x 193 x 84 mm

- Dimensions Heating Zone (L x W) : 120 / 60 mm

- Supply Voltage 230 V / 50/60 Hz

- Power : Heat Zone 200 W Medium Wave Infrared

- Fast and efficient heating

- Safety Class: I

- Regulation: 50°C – 400°C (150°F – 750°F)

- Plug Type: EU