Technical specification of Eppendorf centrifuge 5424R  listed in Call for Proposal #SZTE/2019/PB09693

- Required quantity: 1 pc

- Max. rotor capacity: 24 × 1.5/2.0 mL vessels

- Max. speed: 21,130 × g (15,000 rpm)

- Soft-touch one-finger lid closure for ergonomic operation

- Temperature range from -10 °C to +40 °C

- ECO power shut-off engages after 8 hours of non-use to reduce energy consumption (40 % over night) and extend compressor life (can be deactivated)

- FastTemp function for quick pre-cooling of the centrifuge, e.g. from ~ 21 °C to 4 °C in only 8 minutes

- Guaranteed temperature of 4 °C at maximum rotational speed

- Built-in condensation drain to eliminate water accumulation in the rotor chamber

- weight of centrifuge not more than 21 kg

- OptiBowl® design for quiet operation, even without a rotor lid.

- Short spin centrifugation function at the maximum speed of the inserted rotor

- Rotor and rotor lid are made from aluminum

- Option for setting the time countdown at immediately or just achieve the rotational speed

- Rotational speed adjustable in 50 rpm increments

- Optional rotor not just for standard 1.5 ml columns but also for spin columns

Please upload your offer via the website of the Call for the Proposal, 
No. SZTE/2019/PB09693