Technical specification of Call for proposals No SZTE/2022/PB12189
Calcium Fluoride(CaF2) substrates with Raman grade

- Product name: Calcium Fluoride(CaF2) substrates with Raman grade

- Required quantity: 100 pcs

- Specification:

Substrate Material: Mono Crystal CaF2, Raman grade
Outer Diameter: 13 mm  +/-0.1 mm
Thickness: 0.50,+/-0.025 mm
Surface Quality: Double side polished(DSP), 20-10 S/D
Parallelism: <1 arc min
Flatness: <Lambda/10 @ 633nm over clear aperture (on-jig tested, >85% of surface)
Orientation for Polish: Random
Chamfer: Protection chamfer only; Typ 0.1mmx45deg
Uncoated  
does not interfere with the emission spectral results
Wide transmission range from deep UV to mid IR
Complete flexibility regarding the laser between 130nm to 10 um
One single Raman peak only at 321cm-1 specific to CaF2
No other Raman peaks to confuse the results
Minimal fluorescence
Durable and insoluble
Packing: Plastic box packed - vacuum packed; Maximum 50pcs/box

Formal requirements of the quotations. Please include these data in your proposal:

- Please refer to SZTE/2022/PB12189

- Name of the bidder company, address. Please note that the bidder's company name must be same as that of the contracting and invoicing company (In case the bidder company outsources the invoicing, please notify us before the tender deadline)

- The NET price for the products + all the expenses (e.g. shipping to Szeged, Hungary; handling etc.) of the purchase. Please note that in case shipping/handling cost is not clearly listed in the offer, we will assume that the total cost includes shipping/handling cost, and no further shipping/handling cost can be charged.

- Please list unit prices, subtotals and net total. Please note that quoted net price (subtotals and totals) must match the prices listed in the final invoice to be issued upon delivery. (In case a bidder needs to pay additional taxes, fees, licenses etc after the products/services offered, they are kindly requested to be aware of that and consider these extra cost when calculating the offer net total price.)

- Please do not list the bank charge (to be paid by customer).

- Terms of delivery and delivery/lead time.

- Terms of payment (such as: net 30 or net 60 or prepayment). Our University prefers to pay with wire transfer.

- Quote validity and/or quote issue date. The quotation should be preferably valid for 60 days, due to excessive evaluation period, but at least for 30 days.

  - Quote validity day should be not earlier than the deadline of the procedure.

  - Quote issue date should not be earlier than the starting date of the call for proposals.

- Please include the specification in your offer (so that it can be clearly evaluated whether it matches all the specified features).

Date: 24 of March, 2022.

Please upload your offer on the website of the Call for the Proposals, No SZTE/2022/PB12189