Technical specification of Call for proposals #SZTE/2022/PB12255
for Multi-gas Analyzers

PART No #1:

Name: Multi-gas analyzer

Required quantity: 1 pc

Required Specifications

- Simultaneous, real-time measurement of CO, CO2, H2 concentration in a gas stream,

- The range of analysis is 0-100 V/V% for CO2 and 0-75 V/V % for H2 and CO,

- Low maintenance requirement, no moving parts,

- Capable of continuous operation at least two months, 24/7,

- The maintenance (e.g., cleaning, re-calibration) can be performed by the user following the guidelines received from the producer,

- Low minimum gas flow-rate requirement (not more than 70 ml/min)

- No maximum gas flow-rate limit

- Sampling at near atmospheric pressure (maximum 0.1 bar overpressure requirement for sampling)

- Controlled in an included software, which allows data acquisition,

- Automatic sampling periodically, (preferentially scheduled in the software),

- Quick response time (below 5 minutes for a complete analysis, but a faster response time is preferred)

- Gas flow-rate independent results

Required warranty: at least one year full warranty

Please include shipping cost for this product in your offer

PART no #2:

Name: Multi-gas analyzer

Required quantity: 1 pc

Required Specifications

- Simultaneous, real-time measurement of CO, CO2, H2 CH4 and C2H4 concentration in a gas stream,

- The range of analysis is 0-100 V/V% for CO2 and CO, 0-50 V/V % for H2 and CH4, 0-20 V/V % for C2H4

- Low maintenance requirement, no moving parts,

- Capable of continuous operation at least two months, 24/7,

- The maintenance (e.g., cleaning, re-calibration) can be performed by the user following the guidelines received from the producer,

- Low minimum gas flow-rate requirement (not more than 70 ml/min)

- No maximum gas flow-rate limit

- Sampling at near atmospheric pressure (maximum 0.1 bar overpressure requirement for sampling)

- Controlled in an included software, which allows data acquisition,

- Automatic sampling periodically, (preferentially scheduled in the software),

- Quick response time (below 5 minutes for a complete analysis, but a faster response time is preferred)

- Gas flow-rate independent results

Required warranty: at least one year full warranty

Please include shipping cost for this product in your offer.

Formal requirements of the offers. Please include these data in your proposal:

▪ Please refer to SZTE/2022/PB12255

▪ Remember to fill out the attached Summary Sheet

▪ Name of the bidder company, address. Please note that the bidder company, the contracting company and the invoicing company must be the same. If you have more than one company/subsidiaries please provide the name of the invoicing company. (In case the bidder company outsources the invoicing, please notify us before the tender deadline)
▪ The NET price for the product within one part + all the expenses (e.g. shipping to Szeged, Hungary; handling etc.) in one offer.

- Please note that quoted net price (subtotals and totals) must match the prices listed in the final invoice to be issued upon delivery, regarding the relevant part.

- Please note that in case shipping/handling cost is not clearly listed in the offer, we will assume that the total cost includes shipping/handling cost, and no further shipping/handling cost can be charged.

- (In case a bidder needs to pay additional taxes, fees, licenses etc after the products/services offered, they are kindly requested to be aware of that and consider these extra cost when calculating the offer net total price.)

- Please do not add bank charge as this will be paid by customer.

▪ Terms of delivery and delivery/lead time.
▪ Terms of payment (such as: net 30 or net 60 or prepayment). Our University prefers to pay with wire transfer.
▪ Quote validity and/or quote issue date.

▪ The quotation should be preferably valid for 60 days, due to excessive evaluation period, but at least for 30 days.
▪ Quote validity day should be not earlier than the deadline of the procedure.
▪ Quote issue date should not be earlier than the starting date of the call for proposals.

▪ Please include the specification in your offer (so that it can be clearly evaluated whether it matches all the specified features).
▪ Warranty and guarantee terms if applicable.

Thank you very much in advance for your cooperation!

Version date:21/04/2022.

Please upload your offer via the website of the Call for the Proposal, 
No. SZTE/2022/PB12255