Technical specification of Call for proposals #SZTE/2022/PB12504
for Handheld, battery-powered spectroradiometer and Handheld fluorimeter with integrated Light Meter, GPS module and USB/Bluetooth communication

Required products:

| # | Name: | Required quantity: | Specification: |
| --- | --- | --- | --- |
| 1 | Handheld, battery-powered spectroradiometer | 1 pc | - handheld device - max dimensions: 50x70x30 mm - weight: maximum 75 g - able to measure the light intensity and spectral quality in the range 340 to 850 nm. - Bluetooth communication. - Control and data acquisition: via mobile application. - Rechargeable Li-Ion battery (capacity min 650 mAh) - Battery charging via USB port. Warranty : min 1 year |
| 2 | Handheld fluorimeter with integrated Light Meter, GPS module and USB/Bluetooth communication | 1 pc | - Handheld device - Max dimensions: 135x65x35 mm - Weight: maximum 200 g - Able to measure the following parameters: PAR (400-700 nm), Ft, QY, OJIP, NPQ, and LC. - Sample holder with detachable clips. - Light emitter: blue (470 nm) LED - Saturating pulse illumination: up to 3000 µmol (photon) m-2 s-1. - Actinic illumination: ranged 10-1000 µmol (photon) m-2 s-1. - Measuring illumination: up to 0.09 µmol (photon) m-2 s-1. - Detector type: PIN photodiode with bandpass filters (wavelength range: 667-750 nm). - Data acquisition via USB port and Bluetooth. - Rechargeable Li-Ion battery (capacity min 2000 mAh) - battery charging via USB port. - Warranty : min 1 year. |
| 3 | Please include shipping cost |  |  |
|  | Please include total price (sum of above) |  |  |

Formal requirements of the offers. Please include these data in your proposal:

▪ Please refer to SZTE/2022/PB12504

▪ Name of the bidder company, address. Please note that the bidder company, the contracting company and the invoicing company must be the same. If you have more than one company/ subsidiaries please provide the name of the invoicing company. (In case the bidder company outsources the invoicing, please notify us before the tender deadline)
▪ The NET price for both products + all the expenses (e.g. shipping to Szeged, Hungary; handling etc.) in one offer.

- Please note that quoted net price (subtotals and totals) must match the prices listed in the final invoice to be issued upon delivery.

- Please note that in case shipping/handling cost is not clearly listed in the offer, we will assume that the total cost includes shipping/handling cost, and no further shipping/handling cost can be charged.

- (In case a bidder needs to pay additional taxes, fees, licenses etc after the products/services offered, they are kindly requested to be aware of that and consider these extra cost when calculating the offer net total price.)

- Please do not add bank charge as this will be paid by customer.

- Terms of delivery and delivery/lead time.

- Terms of payment (such as: net 30 or net 60 or prepayment). Our University prefers to pay with wire transfer.

- Quote validity and/or quote issue date.

▪ The quotation should be preferably valid for 60 days, due to excessive evaluation period, but at least for 30 days.
▪ Quote validity day should be not earlier than the deadline of the procedure.
▪ Quote issue date should not be earlier than the starting date of the call for proposals.

▪ Please include the specification in your offer (so that it can be clearly evaluated whether it matches all the specified features).
▪ Warranty and guarantee terms if applicable.

Thank you very much in advance for your cooperation!

Version date: 29/Aug/2022.

Please upload your offer via the website of the Call for the Proposal, 
No. SZTE/2022/PB12504