Technical specification of Call for proposals #SZTE/2022/PB12713 
for Plastic Extrusion Filament Maker with Precision Dryer System

Product name:  Plastic Extrusion Filament Maker with Precision Dryer System

Required quantity: 1 pc

Specification:

Polymer and composite filament maker

Purpose for mixing, compounding & experimentation with composite production

Operational Heating Temp: the minimum of maximal temperature is 450 °C

Ceramic Band heater

At least 4 heating zones

Compatible materials: PLA, ABS, PC, PS, PETG, TPU, TPE, PPS, PVA, Bio PE, NEW PET, PA (6,12, 66), PEEK, PC, PS, PEKK, PAEK, PEI, PSU, PES, PTFE, PVD+,

Independent controls: Yes

Mixing RPM: maximum of the minimum is 2 RPM to minimal of maximum is 15 RPM

Extruded Filament diameter range maximum of minimum 0.5 mm to minimum of maximum is 3 mm (range at 0.02 - 0.12 inches)

Optical sensors for diameter accuracy min. 43 microns (1.69 mils)

Replaceable Nozzle extruder with Diameter 4 mm (0.16 inches)

Extruder system:  at least High chromium and molybdenum steel alloy with nitride Hardening treatment with min. 3 Compression stage with Swappable Extrusion design

Voltage: compatible with 230 V Frequency 50 - 60 Hz system

Hopper volume: min. 2 liters

Extrusion data analysis software

Includes:

- 1x High cleaner – min. 2400gr.

- 1x Mid cleaner – min. 2200gr.

- 1x PLA granulate – min. 2500gr.

- 1x HDPE – min. 6000gr.

- min. 2x Combi Nozzle Package

- min. 2x Puller Wheel

- min. 1x Spool holder

- min. 1x Spool

Polymer Dryer:

Volume: min. 3 liters

Quick-release Assembly

Drying capacity: min. 1kg / 3 hr (for PA6)

Hopper material: at least Stainless steel polished

Compatibility: Granulates and flakes

Recommendation material amount: 1 to 3 kg

Voltage 220 - 230V Frequency 50 - 60 Hz

Adjustable stirring speed 10-15 RPM

Adjustable airflow

Stable temperature control upto min. 150 °C

Support Plan for 2 years which includes:

- Personalized Online Training

- Phone Support

- Replacement Machine

- Priority Email Support

- Extended Hardware Warranty

- Firmwares

Formal requirements of the offers. Please include these data in your proposal:

▪ Please refer to SZTE/2022/PB12713

▪ Name of the bidder company, address. Please note that the bidder company, the contracting company and the invoicing company must be the same. If you have more than one company/ subsidiaries please provide the name of the invoicing company. (In case the bidder company outsources the invoicing, please notify us before the tender deadline)
▪ The NET price for all products within one part + all the expenses (e.g. shipping to Szeged, Hungary; handling etc.) in one offer.

- Please note that quoted net price (subtotals and totals) must match the prices listed in the final invoice to be issued upon delivery.

  - Please note that in case shipping/handling cost is not clearly listed in the offer, we will assume that the total cost includes shipping/handling cost within the Part, and no further shipping/handling cost can be charged.

  - (In case a bidder needs to pay additional taxes, fees, licenses etc after the products/services offered, they are kindly requested to be aware of that and consider these extra cost when calculating the offer net total price.)

  - Please do not add bank charge as this will be paid by customer.

  - please list the net total sum within one Part

- Terms of delivery and delivery/lead time.

- Terms of payment (such as: net 30 or net 60 or prepayment). Our University prefers to pay with wire transfer.

- Quote validity and/or quote issue date.

▪ The quotation should be preferably valid for 60 days, due to excessive evaluation period, but at least for 30 days.
▪ Quote validity day should be not earlier than the deadline of the procedure.
▪ Quote issue date should not be earlier than the starting date of the call for proposals.

- Please include the specification in your offer (so that it can be clearly evaluated whether it matches all the specified features).
Warranty and guarantee terms if applicable.

- Also, please fill in and submit the attached Summary Sheet

Thank you very much in advance for your cooperation!

Version date: 14/Nov/2022.

Please upload your offer via the website of the Call for the Proposal, 
No. SZTE/2022/PB12713