Technical specification of Call for Proposals SZTE/2023/PB12877 for Sound and Vibration Module

Product name: Sound and Vibration Module

Required quantity: 1 pc

Specifications:

- Input channels: 2 (or more), connector: BNC

  - Number of input ranges: min. 6 (from -20 to +30 dB amplification)

  - Maximum sampling frequency: 208,8 kS/s or more

  - Resolution: 24 bit; Delta-sigma ADC

  - Common mode rejection ratio at 0 dB gain: min 80 dBc

  - Dynamic range at 0 dB gain: min 100 dB

  - Total harmonic distortion (THD) at -1 dBFS: better than -95 dBc

  - Intermodulation distortion (IMD) better than -100 dBc (using 14 and 15 kHz components)

- Output channels: 2 (or more) , connector: BNC

  - Differential or pseudodifferential

  - Maximum sampling frequency: 51,2 kS/s or more

  - Resolution: 24 bit; Delta-sigma DAC

  - Attenuation ranges: min. 3

  - Dynamic range at 0 dB attenuation: min 90 dB

  - Total harmonic distortion (THD) at -1 dBFS: better than -80 dBc

  - Intermodulation distortion (IMD) better than -100 dBc (using 14 and 15 kHz components)

- Bus interface: PXI or PXI-e

- Data transfer over DMA

Possible examples

NI PXI-4461, 204.8 kS/s, 2-Input/2-Output PXI Sound and Vibration Module

OR

NI PXIe-4463 51.2 kS/s, 2-Output PXI Sound and Vibration Module AND

PXIe-4464 204.8 kS/s, 119 dB, 6 Gains, AC/DC-Coupled, 4-Input PXI Sound and Vibration Module

Formal requirements of the offers. Please include these data in your proposal:

▪ Please refer to SZTE/2023/PB12877

▪ Name of the bidder company, address. Please note that the bidder company, the contracting company and the invoicing company must be the same. If you have more than one company/ subsidiaries please provide the name of the invoicing company. (In case the bidder company outsources the invoicing, please notify us before the tender deadline)
▪ The NET price for the product + all the expenses (e.g. shipping to Szeged, Hungary; handling etc.) in one offer and the net total sum.

- Please note that quoted net price (subtotals and totals) must match the prices listed in the final invoice to be issued upon delivery.

  - Please note that in case shipping/handling cost is not clearly listed in the offer, we will assume that the total cost includes shipping/handling cost, and no further shipping/handling cost can be charged.

  - (In case a bidder needs to pay additional taxes, fees, licenses etc after the products/services offered, they are kindly requested to be aware of that and consider these extra cost when calculating the offer net total price.)

  - Please do not add bank charge as this will be paid by customer.

  - Please include the net total sum

  - (In case a bidder needs to pay additional taxes, fees, licenses etc after the products/services offered, they are kindly requested to be aware of that and consider these extra cost when calculating the offer net total price.)

- Terms of delivery/delivery lead time

- Terms of payment (such as: net 30 or net 60 or prepayment). Our University prefers to pay with wire transfer.

- Quote validity and/or quote issue date.

▪ The quotation should be preferably valid for 60 days, due to excessive evaluation period, but at least for 30 days.

▪ Quote validity day should be not earlier than the deadline of the tender.

▪ Quote issue date should not be earlier than the starting date of the call for proposals.

- Please include the specification in your offer (so that it can be clearly evaluated whether it matches all the specified features).

- Warranty and guarantee terms if applicable.

Thank you very much in advance for your cooperation!

Version date: 10 of February, 2023.