Technical specification of Call for proposals #SZTE/2023/PB13414
for Plasma Cleaner and Dry Pump

Required product: Plasma Cleaner and Dry Pump

Short description:

The item to be procured is a tabletop plasma cleaner, intended for surface cleaning and sample surface activation. For applications with oxygen gas, the quote must include a dry pump, compatible with the plasma cleaner and suitable for pumping oxygen gas.

Required quantity: 1 pc

Specifications:

Minimum requirements (meeting these requirements must be documented in the offer):

- Compact, tabletop plasma cleaner with viewing window.

- Chamber diameter: at least 7 cm.

- Chamber volume: at least 700 cm3.

- Chamber material: stainless steel, aluminum, Pyrex or equivalent.

- Compatible sample tray made of stainless steel, aluminum, Pyrex or equivalent material.

- Adjustable radio-frequency (RF) power supply with operation frequency either in the kHz or in the MHz range.

- If the cleaner is operated in the MHz range, the peak power is at least 15 W.

- If the cleaner is operated in the kHz range, the peak power is at least 100 W.

- Metering valve/rotameter to control the gas flow and the chamber pressure.

- Compatible valve(s) for gas supply and pump connection.

- Compatible dry pump for oxygen usage (fluid shouldn’t be required for operation).

- All required accessories (adaptors, connectors, hoses, tubing, etc.) for connecting the dry pump to the plasma cleaner.

- Electrical input (for all components): 230 V, 50 Hz, standard EU plug.

- Compatible pressure gauge for monitoring the chamber pressure

+ Please include shipping cost to end user’s address

Formal requirements of the offers. Please include these data in your proposal:

- Please refer to SZTE/2023/PB13414

- Name of the bidder company, address, website. Please note that the bidder company, the contracting company and the invoicing company must be the same. If you have more than one company/ subsidiaries please provide the name of the invoicing company. (In case the bidder company outsources the invoicing, please notify us before the tender deadline)

- Customer name: University of Szeged, Dugonics tér 13. H-6720 Szeged, Hungary, VAT No: HU19308650

- The NET price for the product, accessories (of the required quantity specified above) + all the expenses (e.g. shipping to Szeged, Hungary; handling etc.) subtotals and the net total sum.

  - Please note that quoted net price (subtotals and totals) must match the prices listed in the final invoice to be issued upon delivery., and no further costs can be charged.

  - Please note that in case shipping/handling cost is not clearly listed in the offer, we will assume that the total cost includes shipping/handling cost, and no further shipping/handling cost can be charged.

  - Please do not add bank charge as this will be paid by customer.

  - Please include the net total sum and the relevant currency. Accepted currencies: HUF, EUR, USD, GBP, SEK, CHF, NOK, DKK, CAD, AUD, JPY, CZK.

  - (In case a bidder needs to pay additional taxes, fees, licenses etc after the products/services offered, they are kindly requested to be aware of that and consider these extra cost when calculating the offer net total price.)

  - Magyarországi ajánlattevők figyelmébe/for Hungarian bidders only: Kérjük a nettó és a bruttó részösszegek, az ÁFA és végösszegek feltüntetését is - please indicate net and gross prices and VAT as well

- Terms of delivery /delivery lead time

- Terms of payment (such as net 60 days, or net 30 days or prepayment etc.). Our University prefers to pay with wire transfer.

- Quote validity and/or quote issue date.

  - The quotation should be preferably valid for 60 days, due to excessive evaluation period, but at least for 30 days, if possible.

  - Quote validity day should be not earlier than the tender deadline.

  - Quote issue date should not be earlier than the tender start date.

- Please include the specification in your offer (so that it can be clearly evaluated whether it matches all the specified features)

- Warranty/guarantee terms, if applicable

- Custom tariff code (VTSZ, HTS code)

- In case the product is custom made please include the relevant manufacturer’s declaration

- Year(s) of manufacture of the device(s); also please declare if the device is new

- Please declare if your company is a manufacturer or retailer of the offered product

- Signature and stamp

# Also, please fill out the attached summary sheet, print and sign and stamp it and upload it along with your offer. In case your offer contains all the information that are listed in the summary sheet, you are not required to fill out the  summary sheet, however this sheet is helpful for the suppliers to match the project criteria. We apologize for the inconvenience.

Thank you very much in advance for your cooperation!

Version date: October 3. 2023.

Please upload your offer via the website of the Call for the Proposal, 
No. SZTE/2023/PB13414