Specification — modified

Webra ID: PB13459

Subject: Special optical kit for interferometric multiplexing of KrF amplifier

The special optical kit consists of the following items:

- Quantity: 1 pc.; Size: 90mm x 40 mm x 8 mm; Material: CaF2, Coating: TFP 248nm/56°;
Surface confirmity: < λ/4; Polarization contrast for the TFP coatings: Tp >80%, Ts<2%; maximum homogeneity for bulk material;

- Quantity: 2 pcs.; Size: 75mm x 40 mm x 5 mm; Material: CaF2, Coating: TFP 248nm/56°;
Surface confirmity: < λ/4; Polarization contrast for the TFP coatings: Tp >80%, Ts<2%; maximum homogeneity for bulk material;

- Quantity: 1 pc.; Size: 60mm x 90 mm x 10 mm; Material: Fused silica, Coating: TFP 248nm/56°;
Surface confirmity: < λ/4; Polarization contrast for the TFP coatings: Tp >80%, Ts<2%; maximum homogeneity for bulk material;

- Quantity: 1 pc.; Size: 110mm x 160 mm x 30 mm; Coating: HR248/0°;
Surface confirmity: < λ/4; maximum homogeneity for bulk material;

- Quantity: 2 pcs.; Size: 75mm x 50 mm x 12.5 mm; Coating: HR248/45°;
Surface confirmity: < λ/4; maximum homogeneity for bulk material;

- Quantity: 1 pcs.; Size: 100mm x 60 mm x 8 mm; Material: Fused silica, Coating: HR248/45°;
Surface confirmity: < λ/4; maximum homogeneity for bulk material;

- Quantity: 1 pcs.; Size: 100 mm x 100 mm x 20 mm; Material: Herasil, Coating: HR248/0°;
Surface confirmity: < λ/4; maximum homogeneity for bulk material;