Optikai vonal- és távolságmérők

- Optikai távolságérzékelő – 1 db:

2D/3D – profile sensor

Sensor with integrated controller

Aluminium housing IP65

Measuring range: z-axis: 25 mm / x-axis: 25 mm (MMR)

Measurement points per profile: up to 640

Profile frequency: up to 300 Hz

Light source: 8mW (laser class 2M)

Laser wavelength: 658 nm (red)

Power supply: 11 – 30 VDC, 500 mA (Power over Ethernet available)

Interfaces:

1x Ethernet

1x multifunction port (power supply, digital input, RS422)

Included accessories:

Connection cable for multifunction port (12-pole Escha plug to open ends, 5 m)

External laser safety interlock

- Ethernet kábel – 1 db:

5m, for drag chain use, like SC2600/2900-0,5; with length 5m

- Precíziós optoérzékelő – 1 db:

Méréstartomány: 10 mm

Ethernet/ethercat és RS422 ki/bemenet. 0,03 um felbontás (20 Khz-es mintavételnél) Linearitás: ≤±0,03% FSO

- Precíziós optoérzékelő – 1 db:

Méréstartomány: 20 mm

Ethernet/ethercat és RS422 ki/bemenet. 0,3 um felbontás (20 Khz-es mintavételnél) Linearitás: ≤±0,02% FSO

- Kimeneti tápkábel – 1 db:

Hossz: 3 m

- Tápegység – 1 db:

24 V / 2,5 A, Input 100 – 240 VAC, output 24 VDC / 2,5 A, for snap in mounting on DIN 50022 rail