Specifikáció

| Input Impedance | 1012 Ω, shunted by 1pF |
| --- | --- |
| Output Impedance | 100 Ω, both outputs |
| Gain | X1: ±0.1% |
| Input Voltage Range | ±5 V |
| Risetime | 15 µs, 10-90% |
| Noise Level | 500 μV peak-to-peak* |
| Input Capacitance Compensation | 0-50 pF |
| Gate Leakage Curent | ±10 pA, adjustable to zero |
| Electrode Resistance Test | 1 mV/MΩ |
| DC Positioning | ±300 mV |
| Common Mode Rejection | >104 (in differential mode) |
| Power | Four 9 V alkaline batteries, supplied |
| Dimensions | 8.5 x 3.5 x 2.2 in. (22 x 9 x 6 cm) |
| Shipping Weight | 5 lb. (2.3 kg) |

Features

- Driven guard shield

- Test port

- Ground port

- Portable

- Remote headstage

Benefits

- Cost effective

- Battery powered

- Capacitance compensation