Object: in vivo haemodynamic acquisition system with accessories

Description: The system suitable for blood pressure registration (systolic and diastolic blood pressure) in vivo. The system includes the PowerLab 4/35, a high-performance data acquisition system with transducers for recording blood pressure, the amplifier, the software for evaluation of results, cables for transducers and amplifier, and a homeothermic controller and plate (with rectal probe).

Parameters:

- A high-performance data acquisition system suitable for a wide range of in vivo research applications including physiology, pharmacology and biology that require up to 4 input channels.

- The unit is capable of recording of both in vivo (blood pressure) and in vitro (dose response curve, DMT normalization) traces.

- The homeothermic controller can be used to maintain normal animal body temperature in vivo while performing acute or chronic surgery on a small animal under anesthesia (with resolution of ±0.1°C).

- Compatibility with DMT myograph (Danish MyoTechnology, modell 410A)

Specifications:

PowerLab 4/35 with LabChart Pro

Number of input channels: 4

Amplification range: ±2 mV to ±10 V full scale in 12 steps

Maximum input voltage: ±15 V

Input impedance: ~ 1 MΩ || 100pF

Low-pass filters: 1 Hz – 1 kHz in 2:5:10 steps; 2 kHz, 25 kHz

Maximum sampling rates: 200 kHz on one or two inputs 100 kHz on 3 or 4 inputs

ADC resolution: 16 bit

Quad Bridge Amplifier

Number of inputs: 4

Amplification range: ±200 μV to ±5 V full scale in 14 steps

Maximum input voltage: 12V

Input impedance: 2 x 1 MΩ

Homeothermic Controller and Plate (for rats)

Resolution: ±0.1 °C

Range: + 2-60 °C